ISKANDER MARINE SARL

20' X 9' 6" HIGH CUBE  Refrigerated ISO Cargo Container

CARRIER  1994-1995

Etat  général d’occasion  = révise et répare  perfectement.

— STANDARD FEATURES —

Isolation /Insulation : mousse de polyurethane/ polyurethane foam

Aluminum, steel or sides

Double full opening swing doors on

one end

Aluminum T-grade floors

Most refrigerated machinery features

electrical supply as follows: 208/230,

380/460 Volts AC,

3 phase, 50 or 60 cycle

Isolation/Insulation: mousse de polyurethane foam

Temperature range from -20° F to 70° F

20' Refrigerated

Outdoor Dimensions (mm)

Height   2896

Width  2438

Lenght  6058

Indoor dimensions (mm)

Lenght  5500

Width  2280

Height  2450

| Tare weight | 2,270 Kg | 5,005 lbs |
| --- | --- | --- |
| MGW | 32,800 Kg | 72,324 lbs |
| Payload | 24,100 Kg | 53,140 lbs |
| Top & Bottom rails | Corten |  |
| End Frames | Corten |  |
| Outer panels | Aluminium rivetted |  |
| Underfloor | Corten crossbearers |  |
| Inner roof | Aluminium |  |
| Tee floor height | 43.5mm |  |
| Door outer | Aluminium Plymetal |  |
| Lock rods | 4 |  |
| Goose neck tunnel | Fitted |  |
| Fork lift pockets | No |  |
| Manufacturer | NFF Yundai |  |
| Machine manufacturer | Carrier |  |
| Refrigerant | R134a |  |
| Model | NT40-511 |  |
| Voltage | 415/3ph - 220V Option |  |
| Chart recorder | Option |  |
| Data memory | Yes |  |

ALL NEW CONTAINERS ARE

MANUFACTURED TO THE LATEST ISO

STANDARDS FOR 9 HIGH STACKING

- All dimensions are average. Exact specifications for each model are available.

Isolation/Insulation: mousse /polyurethane foam Isolation/Insulation: mousse de foam

Isolation/Insulation: mousse foam